Outline:

- Memory

- Load/Store instructions

  - Move data between registers and memory

- Data of other types

  - Words, halfwords, and bytes

  - ASCII strings

- Address alignment

- Endianness

Memory

- An Array of bytes

- Each byte is numbered. The number is the address

- Each address identifies a byte

  - If a data item is larger than one byte, its address is the first byte in memory

- A 32-bit address space supports 4Gib

  - A 64-address space supports 16 EiB (exbibytes)

“Address of each byte is written in hex”

Using data in memory:

- Many ISAs like RISC-V cannot compute on data in memory directly

  - Must load data into a register first

    - x1,x2

- Two kinds of instructions to exchange data between registers and memory

  - Load: memory to register

  - Store: register to memory

- Need to know the address to read/write memory

  - You need an address to save/fetch items

Variables defined in your program:

Getting the address of a variable in a register?

- Load a 32-bit constant in a register

- We can also use a pseudo instruction LA

  - It is converted into a couple of real instructions

  - We will learn the real instructions later

Load/Store word instruction

Example:

Each row in the table is a byte

Assume a’s address is in s1. Write RISC-V instructions to do

Int a, b;

b  = a

lw t0, 0(sl)	#To <- a

sw t0, 4(sl)	#To -> b

Array access

Suppose word array A starts from 0x9000 (stored in s1)

A[0], A[1], A[2], … A[i]

We need to specify an address in load/store instructions.

What are the addresses of these words?

Offset is 4*i(index)

If you know of the offset and beginning of array

If index is a small constant, (smaller than 2047)

Asume word array A’s starting (base) address is in register s1.

lw t0, 0(s1)	#A[0]

lw t1, 4(s1)	#A[1]

lw t2, 40(s1)	#A[10]

S2 << 2

4*i

Address alignment

Data items address is a multiple of its size

Address words os a multiple of 4

Address of half words is a multiple of 2

Data address do not have to be aligned in RISC-V, but

Misalignment will cause poor performance

Must be aligned in this course

Align the address of next variable to 2^2 =4

.align 2